# IR Utility Program for the

DVDO line of Video

Processor

Copyright © 2009

Barry V. Gordon

General:

This program was written to explore the IR interface to the DVDO line of video processor. These processors provide a unique IR interface in that it actually supplies two protocols to be used. Using these two protocols it is possible to completely control every aspect of the processor's performance without having to step through menus of choices .

The connection to the DVDO processor may be through standard IR mechanisms using any source of IR  that can handle the protocols.  The Edge processor also supplies a direct hard wired connection on the rear of the unit that accepts “wired IR”.  “Wired IR” is taken to mean the signal that is normally fed to an IR emitter (LED) from an IR transmitting piece of equipment.  I can verify that the signal that comes out of a Xantech amplified connecting block is totally acceptable to the Edge processor, and I suspect that must either sources of IR will be also.  The only issue might be the voltage level used by the IR generator.  Xantech uses 12 volts, many others use 5.

The first protocol is that of the Remote supplied with the unit. It is a standard NEC type protocol having a device and function code.

The second protocol is similar in construction but supplies additional fields to allow for the transmission of discrete information such as the setting (High, Low, Off) for a parameter such as Mosquito Noise;  or a value 25 within a range (-50…0…+50) for a parameter such as Brightness..

Between the above two protocols all of the functionality of the DVDO processos may be dynamically executed as operating conditions change.

Background:

My home theater is controlled by a small and quite unique PC.  It measures 7”x 4”x 8”.  It has no fans, no buttons on the front panel, a start button on the rear and supports two video streams (VGA and DVI at resolutions which include HD at 720p or 1080p), with 5.1 channel audio.  It has a 120 Gig HD, 1 G ram, and clocks an Intel duo core processor at 1.66 GHZ.  The unit is self booting on a power failure runs hot, its case is all fins except for the front and back, and is dead quiet. It can be buried in a cabinet as long as there is sufficient ambient air to cool the case.

When ever I obtain a new piece of equipment for the theater I first put it on my development bench and write a test and utility program for it.  The T&U program is eventually incorporated into the theater’s control system and integrated with  all other aspects of the theater. I normally will not buy a component with only IR capability but the Edge, because of what DVDO did with the IR capabilities was too good to pass up.

My theater's Edge feeds a Cary Cinema 11A audio processor over an HDMI line from it’s HDMI audio out port and a JVC RSA-2 video Projector from the HDMI video out  port.

Disclaimer:

The program is provided as is.  In its off line mode, the program is not able to harm the DVDO Processor as all it supplies are IR patterns to be copied into some remote control.  The patterns are supplied in Pronto Hex format and are generated algorithmically from the protocol specifications.

With regard to the Edge processor; When connected to the Edge  you can cause the Edge to revert to factory defaults but should not be able to damage the Edge in any way providing you have connected it properly using the wired IR interface. By that I mean do not connect the wired IR interface to a source of high voltage. Taking the cable from an IR emitter removing the LED and substituting a mono 1/8” plug is the simple way. A mono 1/8” (3.5mm) male-male cable will also work fine.  Remember the Wired IR jack is polarity sensitive.  Signal is Tip, ground is Sleeve or the long part of the plug.

Program Installation:

This program is supplied as an installable Microsoft system. You just run the setup exe and it will properly install itself.  The program has been tested only under Windows XP SP2.  It has NOT been tested under Vista, however there is no reason it should not be able to run under Vista. It may need to be run under the account of an administrator, but I see no reason why that would be necessary.

If you wish to connect the PC running this program directly to the DVDO Processor you will need to obtain and install a device known as the USB-UIRT which can be obtained from its manufacturer for about $50.  Once this device is installed you will be able to do many interesting IR related things from your PC.

Troubleshooting and Support:

Feel free to contact the author at his eMail address (barry@the-gordons.net)  if you have any difficulty installing or using the program.

Feedback is appreciated.  Criticism with explanation is accepted and appreciated.

The remainder of this document discusses the programs operation.

Program Operation:

When the program starts it checks for the presence of a USB-UIRT. If a USB-UIRT is not found a message is issued and the following screen will be presented. [Note The command to  Send IR data to the processor will only be enabled if you have  a USB-UIRT installed and operating on the PC].

The three radio buttons in the top area control which type of IR pattern will be generated or sent to the DVDO processor.  Clicking on any button will change the contents of the IR command list to be appropriate for that IR protocol. Each button will be discussed in what follows. The manual button is not normally available.  If you require the ability to manually generate IR codes contact the author.

The Gen ALL command button allows you to generate a text file which contains the commands for that protocol in Pronto Hex format.

Discrete Commands that have enumerated options (e.g. On/Off)  will generate the patterns for all enumerations.  Value commands (e.g. Brightness)  will generate patterns for the minimum, middle and maximum values of the range.

The Send command will send the currently selected IR pattern which appears in the right hand pane to the DVDO processor through either a wired connection or an IR emitter/Blaster.  The USB-UIRT has the capability for two independent zones of IR through a stereo jack in the rear, and contains an IR blaster which emits from the front of the unit.

Processor Model Selection:  The next step is to select the model to be used as the target for the IR to be generated. The drop down model list is used toi pick the appropriate DVDO processor model as shown on the next screen

Once the model is selected, the main list box will be populated based upon whether discrete IR or IR from the Remote is to be generated.  The following shows an example after the model has been selected:

Discrete IR Protocol:  Clicking on a command in the command list will bring up the parameters for that command and the screen will look like the following for commands that have enumerated parameters:

If the command clicked on has a range of values the widow will look as follows:

Clicking on the enumerated command option in the first example, or the IR Gen  command in the second option, will produce a screen that has the right hand IR pane filled in with the Pronto HEX pattern for that specific command.  This command may be cut and pasted as needed.

If you double click on the command option then the system will send the IR pattern just as if you had clicked on the send command.

Remote IR Protocol:  If the remote IR protocol is selected the window appears as follows with the same general rules applying as applied for the Discrete protocol.